Phase B – Kitchen Timer (Beta Release)

Evan Hill

ENCE 4231

Embedded Systems Programming

Dr. Goncalo Martins

Project Requirements

When developing ideas for the kitchen timer, examples were sought out on the internet to find possible features to be included in the design. Examples of these were the capability to be stood up or mounted, be solar-powered, using dials or buttons to control the timer, and being able to control the volume. For the kitchen timer, several requirements and constraints were decided on that needed to be met with the design. Starting off, the total size of the PCB and enclosure must be within 10 cm. by 10 cm. It would run solely on an external power source through a USB cable. Two user buttons would be implemented to make full use of the product. The kitchen timer will use a 4 digit, 7 segment display to display the timer count down. The kitchen timer would be capable of increasing the time before the count down, starting, pausing, and resetting the time. After the kitchen timer reaches a time count of 0, it will then buzz to alert the user. As for the enclosure, the size requirements remain the same as described. The user should have access to all buttons, the USB port and the display.

System Design

Figure 1 – PCB Design Block Diagram

Above depicts the diagram developed with the ideas for the requirements and constraints for the kitchen timer PCB. At the center of everything is the microprocessor, which handles the computations and controls all the peripherals. There are 3 user buttons to control the functions of the kitchen timer, and LEDs to indicate the status of the kitchen timer. A buzzer is implemented to indicate when the time has reached 0, and an alarm flag is being raised. A USB driver is used to provide power to the kitchen timer, alongside voltage regulator to provide 3.3 V for those peripherals which require 3.3 V rather than 5 V. For COMMS, an ESP8266 was selected to provide the capabilities to interact with external devices through Wi-Fi. The ISP provides a port to program the kitchen timer. The crystal provides a clock pulse at 16 MHz for the microprocessor. And finally, the 4 digits, 7 segment display is used to provide a visual indication of the current time.

Components Selection

| Item | Qty | Reference(s) | Value | LibPart | Footprint |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | C2, C3 | 22 pF | Device:C_Small | Capacitor_SMD:C_0603_1608Metric |
| 2 | 7 | C4, C5, C6, C7, C10, C11, C12 | 0.1 uF | Device:C_Small | Capacitor_SMD:C_0603_1608Metric |
| 3 | 1 | C8 | 10 uF | Device:C_Polarized_Small_US | Capacitor_SMD:C_0805_2012Metric |
| 4 | 2 | C9, C13 | 1 uF | Device:C_Small | Capacitor_SMD:C_0603_1608Metric |
| 5 | 1 | C14 | 2.2 uF | Device:C_Small | Capacitor_SMD:C_0603_1608Metric |
| 6 | 1 | C15 | 10 nF | Device:C_Small | Capacitor_SMD:C_0603_1608Metric |
| 7 | 3 | D1, D2, D3 | LED | Device:LED | LED_SMD:LED_0805_2012Metric |
| 8 | 1 | F1 | PTC SMD | Device:Polyfuse_Small | Fuse:Fuse_1812_4532Metric |
| 9 | 1 | J1 | AVR-ISP-6 | Connector:AVR-ISP-6 | Connector_PinSocket_2.54mm:PinSocket_2x03_P2.54mm_Vertical |
| 10 | 1 | J2 | USB_B_Mini | Connector:USB_B_Mini | Connector_USB:USB_Mini-B_Lumberg_2486_01_Horizontal |
| 11 | 1 | J3 | ESP_Conn | Connector_Generic:Conn_02x04_Odd_Even | Connector_PinSocket_2.54mm:PinSocket_2x04_P2.54mm_Vertical |
| 12 | 1 | LS1 | Speaker | Device:Speaker | Buzzer_Beeper:Buzzer_12x9.5RM7.6 |
| 13 | 2 | R1, R2 | 10k | Device:R | Resistor_SMD:R_0805_2012Metric |
| 14 | 2 | R3, R4 | 330 | Device:R | Resistor_SMD:R_0805_2012Metric |
| 15 | 8 | R5, R6, R7, R8, R9, R10, R11, R12 | 100 | Device:R_Small | Resistor_SMD:R_0805_2012Metric |
| 16 | 1 | R13 | 10k | Device:R | Resistor_SMD:R_0603_1608Metric |
| 17 | 2 | R14, R15 | 22 | Device:R | Resistor_SMD:R_0603_1608Metric |
| 18 | 1 | R16 | 1k | Device:R | Resistor_SMD:R_0805_2012Metric |
| 19 | 1 | R17 | 10k | Device:R_Small | Resistor_SMD:R_0603_1608Metric |
| 20 | 2 | S1, S2 | PTS125SM43SMTR21M_LFS | PTS125_SMD_Button:PTS125SM43SMTR21M_LFS | PTS125_SMD_Button:PTS125_SMD_Button |
| 21 | 1 | S3 | PTS526_SM08_SMTR2_LFS | PTS526_SMD_Button:PTS526_SM08_SMTR2_LFS | PTS526_SMD_Button:PTS526_SMD_Button |
| 22 | 1 | U1 | CA56-12EWA | Display_Character:CA56-12EWA | Display_7Segment:CA56-12EWA |
| 23 | 1 | U2 | 74HC595 | 74xx:74HC595 | Package_SO:TSSOP-16_4.4x5mm_P0.65mm |
| 24 | 1 | U3 | ATmega32U4-A | MCU_Microchip_ATmega:ATmega32U4-A | Package_QFP:TQFP-44_10x10mm_P0.8mm |
| 25 | 1 | U4 | USBLC6-2SC6 | Power_Protection:USBLC6-2SC6 | Package_TO_SOT_SMD:SOT-23-6 |
| 26 | 1 | U5 | LP2985-3.3 | Regulator_Linear:LP2985-3.3 | Package_TO_SOT_SMD:SOT-23-5 |
| 27 | 1 | Y1 | 16 MHz | Device:Crystal_GND24 | Crystal:Crystal_SMD_Abracon_ABM8G-4Pin_3.2x2.5mm |

Table 1 – Component Table

Build Prototype

Figure 2 – Kitchen Timer Arduino Shield Prototype 3D Render

A PCB shield Kitchen Timer for the Arduino Uno was employed for the purposes of testing and developing the code for the kitchen timer while the PCB boards were being manufactured offsite. Depicted above is a 3D rendering of the PCB shield used. Compared to the final PCB design, there are some similarities and some differences. Like the final design, a 4 digit 7 segment display is used to display the time, and a buzzer is used to play the alarm signal. Two user pushbuttons are used to control the kitchen timer functionalities, as well as 2 LEDs to indicate the kitchen timer’s status. However, what’s different is rather than a SMD shift register, a simple 2x8 IC socket is used instead to implement a shift register IC. All the capacitors are missing, as well as the ISP, Crystal, Voltage regulator, and ESP8266, which are either not present, or already included in the Arduino Uno. Pin sockets are provided so that the shield can be mounted directly on top of the Arduino Uno for seamless programming.

PCB Design

For the purposes of designing the PCB, KiCad was used to develop the circuit schematics, PCB Design and PCB layout.

Figure 3 – PCB Circuit Schematic made in KiCad

Figure 4 – PCB Layout Mockup

Before beginning designing the layout of the PCB board, I wanted to mockup a simple design for the main peripherals so that I could better direction myself while placing each of the components. The display needs to be in the corner to provide plenty of space for the other components. The USB needs to be on the opposite side from the user buttons so that the USB doesn’t interfere with interacting with the user buttons. The user buttons should be close to each other as well as on the edge for ease of use. And the buzzer also occupies a large space, so placing it far away in the corner provides space for other components.

Figure 5 – PCB Layout with routing

Depicted above is the final PCB layout. Compared to the original mockup, the display, user buttons, and USB remain in similar places. However, while routing, the buzzer needed to be relocated so that the shift register, along with all its resistors, could have a close position to the display. Thankfully, there was ample space beside the buttons to be placed alongside the LEDs to centralize all of the indicators. The microprocessor needed to be placed in the most central location possible, since it would have the most routings to it. Beyond the placement of the rest of the peripherals, they were decided based on the space needed, as well as the ease of routing.

Figure 6 – 3D Render of final PCB Design in SolidWorks

Assembly Stage

Documentation of the assemble process

Software Development

Code formatted according to the mutex – Embedded C Style Guidelines

Block Diagrams and/or flow charts

Enclosure Design

CAD Designs

3D Printed Models